21 Nov, 2025

Lab Project

CSE 360

Experiment Name: Make a Simple Calculator with basic operations like Addition, Subtraction, Multiplication and Division.

Process:

Using 8086 assembly language, this project creates a basic command-line calculator that can be used with the EMU8086 emulator.

One of the four fundamental arithmetic operations is carried out by the calculator once the user enters two integers:

● Addition

● Subtraction

● Multiplication

● Division

Algorithm:

- Start the program.

- Initialize the Data Segment.

- Display a message asking for the first number.

- Call the number-input procedure to store the number.

- Display a message asking for the second number.

- Call the number-input procedure to store the number.

- Show the menu of operations:

- 1 → Addition

- 2 → Subtraction

- 3 → Multiplication

- 4 → Division

- Take the user’s choice as a character input.

- Compare the input with the valid options.

- According to the choice:

- Perform addition → store the result.

- Perform subtraction → store the result.

- Perform multiplication → store the result.

- Perform division → store the result.

- If the choice is invalid, display “Invalid choice!”

- Display “Result:” followed by the computed value.

- End the program.

Implementation:

Source Codes:

; You may customize this and other start-up templates;

; The location of this template is c:\emu8086\inc\0_com_template.txt

org 100h

.model small

.stack 100h

.data

msg1 db 10,13,'Enter first number: $'

msg2 db 10,13,'Enter second number: $'

msg3 db 10,13,'Choose operation:',10,13,'1. Addition',10,13,'2. Subtraction',10,13,'3. Multiplication',10,13,'4. Division',10,13,'Choice: $'

msg4 db 10,13,'Result: $'

msg5 db 10,13,'Invalid choice!$'

newline db 10,13,'$'

num1 dw ?

num2 dw ?

result dw ?

.code

main proc

mov ax, @data

mov ds, ax

; First number input

mov ah, 9

lea dx, msg1

int 21h

call input_number

mov num1, bx

; Second number input

mov ah, 9

lea dx, msg2

int 21h

call input_number

mov num2, bx

; Operation choice

mov ah, 9

lea dx, msg3

int 21h

mov ah, 1

int 21h

cmp al, '1'

je addition

cmp al, '2'

je subtraction

cmp al, '3'

je multiplication

cmp al, '4'

je division

; Invalid choice

mov ah, 9

lea dx, msg5

int 21h

jmp exit

addition:

mov ax, num1

add ax, num2

mov result, ax

jmp display_result

subtraction:

mov ax, num1

sub ax, num2

mov result, ax

jmp display_result

multiplication:

mov ax, num1

imul num2

mov result, ax

jmp display_result

division:

mov dx, 0

mov ax, num1

idiv num2

mov result, ax

jmp display_result

display_result:

mov ah, 9

lea dx, msg4

int 21h

mov ax, result

call print_number

exit:

mov ah, 4ch

int 21h

main endp

; Input number procedure

input_number proc

mov bx, 0

input_loop:

mov ah, 1

int 21h

cmp al, 13

je input_done

sub al, '0'

mov cl, al

mov ch, 0

mov ax, bx

mov dx, 10

mul dx

add ax, cx

mov bx, ax

jmp input_loop

input_done:

ret

input_number endp

; Print number procedure

print_number proc

mov cx, 0

mov bx, 10

cmp ax, 0

jge positive

neg ax

push ax

mov ah, 2

mov dl, '-'

int 21h

pop ax

positive:

mov dx, 0

div bx

push dx

inc cx

cmp ax, 0

jne positive

print_loop:

pop dx

add dl, '0'

mov ah, 2

int 21h

loop print_loop

ret

print_number endp

end main

ret

# Method:

## Taking Input

The program asks the user to enter two numbers.

A custom procedure input_number reads multi-digit numbers by:

  - Taking input one character at time

  - Converting from ASCII to integer

  - Building the final number using multiplication by 10

The result is stored in BX and then saved in num1and num2.

## Operation Selection

The user selects one of the operations by pressing:

- Addition

- Subtraction

- Multiplication

- Division

The program reads a single character using int 21h(AH = 1) and matches it with the options using cmp and je.

## Performing the Operation

Depending on the choice:

  - Addition: ADD AX, num2

  - Subtraction: SUB AX, num2

  - Multiplication: IMUL num2

  - Division:

    - Clear DX

    - IDIV num2

The result is stored in the variable result.

Result:

The application displays as like user friendly calculator

Conclusion:

This project effectively illustrates how simple arithmetic operations can be carried out using assembly language.

This calculator application teaches us:

- Using interruptions to handle user input

- Putting multi-digit numeric input into practice

- Using CPU registers to do arithmetic

- Using bespoke processes to display numbers

- Conditional branching in menu-based systems

The project offers a solid basis for comprehending low-level programming ideas and the direct hardware execution of tasks.